ISO TC 188/SC  N

Date:   2004-04-14

ISO/WD 12215-7

ISO TC 188/SC /WG 18

Secretariat:   SIS

Hull construction - Scantlings — Multihulls

Construction de la coque -Echantillonnage — Multicoques

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Originator:  ISO/TC 188/WG 18 Scantlings
Convenor:
Mr. Grégoire DOLTO
e-mail: dolto@fin.fr

For the draft reader:

The ISO time schedule pushes WG 18 of TC 188 to produce a CD, whereas the contents of the drafts are still preliminary and open to discussion. This CD is therefore mainly set of articles listing the items to be considered and proposing methods of analysis, rather than a comprehensive draft. The details are, in many cases, still to be implemented or developed, this will be done for  the DIS.

The aim of this CD is to have the international ISO community to validate the general structure of the draft rather than approved a finished draft.

The parts underlined in blue are comments or point to discuss and will be deleted for CD circulation.

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manger of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents	Page

Foreword	iv

Introduction	v

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

4	Symbols	3

5	General	3

6	Design stress and allowable deflexion	3

6.1	Design stresses for metal	3

6.1.1	Basic design stresses	3

6.1.2	Combined design stresses for metal	4

6.2	Design stress for FRP or wood elements	4

7	Calculation of local loads	5

7.1	General	5

7.2	Pressure coefficients	5

7.3	Wet deck pressure determination for catamarans	6

8	Calculation of global loads due to rig and heeling	6

8.1	General	6

8.1.1	Shear force and bending moment on the mast main beam.	6

9	Calculation of global loads due to the sea	8

9.1	General	8

9.2	Bending moment on each hull	8

9.3	Twisting moment T	8

Annex A (normative)  Example of Mast beam calculation under mast compression	9

Annex B (normative)  Example of Twisting moment analysis with differential deflection of crossbeams	11

B.1	Theory	11

B.2	Worked example	12

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies

International Standard ISO 12215 was prepared by Technical Committee ISO/TC 188, Small craft.

Beside this seventh part, ISO 12215 consists of

- Part 1: Materials - Thermosetting resins, glass fibre reinforcement, reference laminate

- Part 2: Materials - Core materials for sandwich construction, embedded materials

- Part 3: Materials - Steel, aluminium, wood, other materials

- Part 4: Workshop and manufacturing

- Part 5: Design pressures for monohull, design stress, scantlings determination

- Part 6: Structural arrangements and details

- Part 8: Rudders

- Part 9: Appendages and rig attachments

The development of ISO 12215 parts 1 to 9 owes a considerable debt to the energy and work of Mr Fritz HARTZ who was involved at the start of the project and was the convener of TC 188 WG 18 until his death on the 16th of November 2002. All the members of WG 18 and TC 188 wish to express their gratitude  for his major contribution to the production of this international standard

Introduction

The reason underlying the preparation of this International Standard is that standards and recommended practices for loads on the hull and the dimensioning of small craft differ considerably, thus limiting the general world wide acceptability of boats.

The objective of this standard is to achieve an overall structural strength that ensures the watertight and weathertight integrity of the craft.

The working group considers this standard to have been developed applying present practice and sound engineering principles. The design pressures of this standard shall be used only with the equations of this standard.

Considering future development in technology and boat types, and small craft presently outside the scope of this standard, provided methods supported by appropriate technology exist, consideration may be given to their use provided equivalent strength to this standard is achieved.

The dimensioning according to this standard is regarded as reflecting current practice, provided the craft is correctly handled in the sense of good seamanship and equipped and operated at a speed appropriate to the prevailing sea state.

Hull construction - Scantlings — Multihulls

# Scope

This part of ISO 12215 applies to determination of design loads, pressures, stresses, and to the determination of the scantlings, including internal structural members of multihull small craft constructed from fibre reinforced plastics, aluminium or steel alloys, wood or other suitable boat building material, with a length of the hull (LH) according to ISO 8666 of up to 24 m. It only applies to intact boats.

The assessment shall generally include all parts of the craft that are assumed watertight or weathertight when assessing stability, freeboard and buoyancy according to ISO 12217, all structural integral parts, and in addition any highly loaded areas like attachment areas of ballast keels, centreboards, rudders, chain plates, etc.

This part of ISO 12215 shall be used in conjunction with Part 5 for general scantlings and local loads. Part 6 shall be used for details, Parts 8 for rudders and part 9 for appendages and rig attachment shall also be used.

NOTE 1	Scantlings derived from this International Standard are primarily intended to apply to recreational craft including charter vessels.

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO 8666:— Small craft — Principal data

ISO 12215-3:— Small craft — Part 3: Materials — Steel, aluminium, wood, other materials

ISO 12217:— Small craft — Stability and buoyancy assessment and categorisation

# Terms and definitions

For the purposes of this International Standard, the following terms and definitions apply.

3.1
design categories

sea and wind conditions for which a boat is assessed by this International Standard to be suitable, provided the craft is correctly handled in the sense of good seamanship and operated at a speed appropriate to the prevailing sea state.

3.1.1
design category A ("ocean")

category of boats considered suitable to operate in seas with significant wave heights above 4 m and wind speeds in excess of Beaufort Force 8, but excluding abnormal conditions, e.g. hurricanes.

3.1.2
design category B ("offshore")

category of boats considered suitable to operate in seas with significant wave heights up to 4 m and winds of Beaufort Force 8 or less

3.1.3
design category C ("inshore")

category of boats considered suitable to operate in seas with significant wave heights up to 2 m and a typical steady wind force of Beaufort Force 6 or less

3.1.4
design category D ("sheltered waters")

category of boats considered suitable to operate in waters with significant wave heights up to and including 0,30 m with occasional waves of 0,5 m height, for example from passing vessels, and a typical steady wind force of Beaufort 4 or less

3.2
design category factor

fw

factor lowering requirements according to design category, its values are according to Table 1

Table 1 — Values of design category factor

| Design Category | A | B | C | D |
| --- | --- | --- | --- | --- |
| Value of fw | 1 | 0,9 | 0,75 | 0,5 |

3.3
loaded displacement mass m LDC

mass of the craft, including all appendages, when in the fully loaded ready for use condition as defined in ISO 8666.”

3.4
sailing craft

boat for which the primary means of propulsion is by wind power, having a total profile area, As as defined in ISO 8666, expressed in m², of all sails that may be set at one time when sailing closed hauled of			 As > 0,07( m LDC)2/3

In the rest of this International Standard, non-sailing craft are called motor craft.

3.5
catamaran

3.6
trimaran

# Symbols

Unless specifically otherwise defined, the symbols shown in Table 1 are used in this International Standard.

- Table 2 — Symbols, coefficients, parameters

| Symbol |  | Unit | Designation/Meaning of symbol |  | Reference/Article concerned |
| --- | --- | --- | --- | --- | --- |
| Principal data |  |  |  |  |  |
|  |  |  | To be implemented |  |  |

# General

The scantling determination shall be accomplished as follows:

- for design stresses and deflexion section 6 shall be used

- for local loads section 7 shall be used

- for global loads due to rig and heeling section 8 shall be used

- for global loads due to efforts due to twisting and bending in the sea section 9 shall be used

# Design stress and allowable deflexion

## Design stresses for metal

### Basic design stresses

For the design tensile, compressive, and flexural loads shall be the smallest of

σd=0,5 σu or 0,9 σy where relevant                                                                               (N/mm²)	(1)

and

τd=τu2                                                                                                                              (N/mm²)	(2)

where

- σdis the design tensile, compressive, and flexural stress 							(N/mm²)

- σu is the ultimate tensile, compressive, and flexural stress 						(N/mm²)

- σy is the yield tensile , compressive, and flexural stress 							(N/mm²)

- τdis the design shear stress 													(N/mm²)

- τuis the design shear stress 													(N/mm²)

NOTE	To be consistent with parts 5 and 8 the design stresses of are high. To take this fact into account, the actual loads are raised by an adequate dynamic factor.

### Combined design stresses for metal

For metallic elements, equation (3) shall be fulfilled in any point of the stock:

σ²+3τ²≤ σd	(3)

## Design stress for FRP or wood elements

For FRP or wood elements, equation (4) shall be fulfilled:

σσu2+ττu2<0,25                                                                                                                                (4)

where σu and τu  are as previously defined

# Calculation of local loads

## General

The scantlings determination of multihulls shall be determined by multiplying, where relevant, the pressure values of monohulls for the same type of craft (motor craft or sailing craft) and the same location (bottom, side, deck superstructure) by the relevant coefficient given in Table 3.

For wet decks and cross-beam faces the pressure determination follows specific consideration.

Key

1  Bottom area	  2  Side outer			3  Side inner		
4  Wet deck		 5 Front facing cross-beam	5  Aft facing cross-beam

Figure 1 — General dimensions of a catamaran

## Pressure coefficients

Table 3 — Values of Multihull pressure coefficient for Power and sailing multihull.

| Multihull Element | Pressure ratio monohull/multihull |  |
| --- | --- | --- |
|  | Cmm | Cms |
|  | Motor craft | Sailing craft |
| Bottom shell |  |  |
| Outer Side shell |  |  |
| Inner Side shell |  |  |
| Deck |  |  |
| Superstructures |  |  |

The convener will propose values for table 3 in May 2004, after discussion with multihull bilders

Comments: The convener received several comments on Part 5, complaining that lower boundary the pressure reduction factor was too high compared to actual boats (side but also on deck and superstructure). It seems, incidentally there is a mistake in Part 5, as this lower bound is 0,4 for sandwich core and 0,25 for strength (in aft part) for side and bottom, whereas it is 0,6 for deck and superstructure where the is much less slamming. If this problem is solved for monohulls, it would be easier to define eventual reduction coeffs for multihulls. Another point is that multihull do not heel, so they could be treated like motorboat monohulls.

Possible pressure reduction for aft topsides.

## Wet deck pressure determination for catamarans

This is an important subject. It seems that below a certain height (about 75 cm for a 12 m Lh cat) the pressure raises "exponentially", at least between 30 to 50 % of Lh from stem. In other areas, something like the side pressure would be OK. The connecting junction between wet deck and inner sides seems to need the same treatment. It is too early to propose an equation, the French industry will propose something in may

Function of

h above WL and ratio

DNV= 2,6 K (D/A)1/3 ncg (1-Hc/Hl)* L/D^2/3

HC= height above WL

HL= 0,22(Kc-(0,8/1000)L)

Difference between  fore half and the rest

The connecting parts between the wet deck (horizontal) areas and the inner shell (near vertical) shall be subject to wet deck pressure.

# Calculation of global loads due to rig and heeling

Comment : The Article 8 and 9 below are very preliminary, the French WG is working on proposals available in May. the aim is to consider any possible global loads, but only when really needed.

## General

The loads due to equilibrium of forces and moments between aerodynamic, hydrodynamic and stability effects induce structural loads.

Among these loads, the main effects are:

  - Compressive load under the mast step or pillar; as defined in 12215-9

  - Tensile loads around main shroud chainplates; as defined in 12215-9

  - Bending moment and shear force due to the effect of the above loads and gravity loads.

The stresses induced by these loads shall not be greater than the basic or combined stresses, as relevant, defined above.

The analysis of the mast beam for a single mast arrangement is considered in article 8.1.1 below. Multi mast arrangements shall be analysed similarly.

Finite element analysis may be used for the global loads.

### Shear force and bending moment on the mast main beam.

Remark: a much finer analysis may be developed, this is very preliminary.

The analysis shown below is the typical analysis to be used corresponding to Figure 2. It is simplified as the boat weight effects are neglected.

Other technical arrangement to feed the mast are allowed, provided the equilibrium between the mast and shroud loads in shear and bending are achieved using sound engineering methods.

The load on the mast step FCMas defined in 12215-9

The design load on the end of the beam is FCMconsidered as the load induced by the transversal rig plus ½ of the longitudinal rig load.

The shear load varies from +FCM to 0 when the mast step load FCMis introduced. But as this must be symmetrical, the shear load is constant.

The transmission of the shear load from the shroud chainplate need to be verified. It may pass directly by a bulkhead, but if there is an opening, like on Figure 2, there is a stress concentration around the opening and secondary bending moment. The shear load may also pass through the hull sides to another bulkhead or the mast bulkhead, but the load path and transmission shall be achieved.

The bending moment varies from 0 to MMB MAX=FCM bC

The shear stress  and bending stress  shall not be respectively greater than d and d, combined where relevant.

An example of such a mast beam analysis is given in Annex A.

Figure 2 — Schematic mast beam section, Shear load and bending moment

# Calculation of global loads due to the sea

## General

The boat is considered:

- For vertical bending moment of the hulls  (hogging / sagging) in a swell perpendicular to the boat axis

- For horizontal bending moment of the hulls ( lateral pressure, etc)

- For twisting moment in a swell 45° to the boat axis. Either for boats with separate crossbeams (like sports cats) or for box-type structure

- For horizontal shear load and bending moment for boats with separate crossbeams (like sports cats)

- etc

## Bending moment on each hull

MBH=2,5 ⋅10−3⋅LWL2⋅∇DLWL2 	(According to BV, to be checked, or according to DNV)

Where DLWL2 is the depth at mid WL (height between bottom of canoe body to sheeline)

The bending stress  shall not be greater than d, at any point of the hull under this Moment.

The above requirement needs to be verified only if D LWL2LWL≤K1 		K1 to be defined

## Twisting moment T

T=Δ L (1+ncg)4		is the twisting moment when the boat is in diagonal swell		(Nm) 	(  )

According to DNV, BV formula seems complicated, and does not consider motor craft.

Where

- ncg =: 1 for sailing multihulls;

- ncg is as defined in 6.1.2 or 12215-5

stress  shall not be greater than d, at any point of the hull under this Moment.

An example of twisting moment calculation using differential deflection of cross-beams is given in Annex B.

Other methods may be used, including FEM, provided sound engineering methods are applied.

When openings are cut in the cross-beams, or any panel subject to significant shear flow or shear stress, care shall be taken to consider the raise of shear stress and the induced secondary bending moment.

- (normative)

Example of Mast beam calculation under mast compression

To be implemented

- (normative)

Example of Twisting moment analysis with differential deflection of crossbeams

  - Theory

Figure B 1 — Scheme of platform twist

When a boat twist, the strain energy  is i WD = ½ T   (Work done!) = SE

SE in bending Ub=6 E I z²L3=6 E I x²θ²L3

SE in torsion Ut=12G Jθ²L

Or θ=T12 EiIi x2L3+GiJiL

Where the ‘GJ’ term may be neglected for ‘open’ section or if a conservative solution is required.  The shear strain energy is negligible for a catamaran with just two or three beams, but for a ‘double bottom’ style wet deck + dry deck + girders extending over 50% of the LWL this might not be the case. So providing the full equation might be helpful for boats, which are marginal in meeting the standard.

This distance of the centre of twist from the origin used for x is:

x=12 EI x∗L312 EIL3 																			(m)    (B1)

where

- T is the Twisting moment defined in equation (  ) 											(Nm)

- Ei Iiis the flexural rigidity of each cross beam

- Gi Jiis the torsional rigidity of each cross beam

- L is the transversal distance between hulls, considered constant (mean value may be used) 	(m)

- x∗ is the longitudinal distance, from an arbitrary datum, between the cross-beam centroid and centre of twist.

- x=x∗−x  where xis the longitudinal distance, from the same datum, of the centre of twist.

With a cantilever beam with a Force F at the end of its length L, the deflection is  FL33 EIand the moment is Fl

From  θ , one can know the bending moment and shear force for each cross-beam at a distance x

M=FL=6 E I θ x L2is the bending moment and F=2ML=6 E I θ x L3is the shear force.

The cross beam can then be analysed as in Annex A

NOTE	The twist formula could be corrected to include the shear stiffness of the cross beams (i.e. GJ/L) but this normally only affects the twist by a few per cent and the complication is unjustified.

  - Worked example

To be implemented